Summary Discussion Leader: ___________________________ Attendance Sign- In: ____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Our company has an Assured Grounding program as a means to protect ourselves against accidental electrical shock.

Guide for Discussion

Program Components: Have the company written policy on file. Our policy is located _____________________________ Have a competent person conduct all tests. Our competent person(s) are: ______________________________________ Test all electrical equipment for proper grounding. Remove any defective equipment from use and tag it to prevent future use. Color code all equipment tested to insure complete test result. We use the following colors: - Winter - - Spring - - Summer - - Fall - A color chart is located_________. Tests: Test for the continuity of the grounding conductor. Test before the equipment is first used; after any repair; after any possible damage and a minimum quarterly (i.e., every three months). Inspections: Visually daily for defects before use. Inspect the following types of equipment: Power Tools, Extension Cords and Temporary Receptacle Boxes

Additional Discussion Notes:

Three prong grounding testers to check extension cord continuity are located ____________________________________________________ __________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Remember

The use of an Assured Grounding Program is not only required, but it is good common sense. Electrocution is no laughing matter and all steps we can take to reduce our exposure to this hazard makes sense.