Each lift director must:

- Be a current qualified rigger as outlined in WAC 296-155-53306 Rigger qualifications; and

- Be a current qualified signal person as outlined in WAC 296-155-53302 Signal person qualifications.

Qualification requirements. Each lift director must:

- Know and understand the relevant requirements of WAC 296-155-53401(6) Duties of assigned personnel;

- Know and understand the relevant requirements of WAC 296-155-53408 Power line safety;

- Know and understand the relevant requirements of WAC 296-155-56420 Operator certification—Written examination—Technical knowledge criteria; and

- Demonstrate that they meet these requirements of this subsection through the successful passing of a written or oral test, containing questions based on (a) through (c) of this subsection. All tests must be documented and provided upon request.

Qualification period. A lift director qualification may not exceed a five-year period; this qualification must be renewed every five years to ensure lift directors maintain qualified status. This renewal must include a documented written or oral exam.

The lift director's duties would include the following:

- Being present at the job site and overseeing the lifting operations;

- Review and discuss the plan in a meeting with all workers who will be involved with the operation.

- Stopping crane operations if alerted to an unsafe condition affecting those operations;

- Ensuring that the preparation of the area needed to support crane operations has been completed before crane operations commence;

- Ensuring necessary traffic controls are in place to restrict unauthorized access to the crane's work area;

- Ensuring that personnel involved in crane operations under-stand and are correctly performing their assigned duties, and under-stand the associated hazards;

- Addressing safety concerns raised by the operator or other personnel, and deciding if it is necessary to overrule those concerns and direct((s)) crane operations to continue. In all cases, the manufacturer's criteria for safe operation and the requirements of this chapter and any other applicable safety and health standards must be adhered to;

- Assigning qualified signal person(s) and conveying that in-formation to the crane operator;

- Ensuring that signal persons assigned meet the qualification requirements located in WAC 296-155-53302;

- Allowing crane operation near electric power lines only when the requirements of WAC 296-155-53408, and any additional requirements determined by the site supervisor, have been met;

- Ensuring precautions are implemented when hazards associated with special lifting operations are present. Such operations include, but are not limited to((, the following)):

  - Multiple crane lifts;

  - Multiple load line lifts;

  - Lifting personnel;

  - Pick and carry operations;

  - Mobile/articulating cranes operating on barges((.)); and(vi) Utilizing vacuum lifters, close proximity lifting magnets, or material handling grapples;

  - Ensuring that the applicable requirements of WAC 296-155-547 through 296-155-55405 are met when lifting personnel;

  - Informing the crane operator of the weight of loads to be lifted, as well as the lifting, moving, and placing locations for these loads;

  - Obtaining the crane operator's verification that this weight does not exceed the crane's rated capacity;

  - Ensuring that a crane's load rigging is performed by a qualified rigger as defined in WAC 296-155-53306;

  - Ensuring that the load is properly rigged and balanced before it is lifted more than a few inches)) Prohibiting cranes/equipment from hoisting when the wind speed exceeds the crane's/equipment's maximum wind speed limitation. A wind speed direction indicator shall be used;(p) Taking the effects of dynamic loading into consideration prior to using the crane/equipment for a hoisting activity.

Additional Topics

When a local storm warning has been issued, the ((competent person)) site supervisor and lift director must determine whether it is necessary to implement manufacturer recommendations for securing the crane/((derrick)) equipment.

During the assembly, disassembly, or reconfiguration of a tower crane, the lift director may not perform more than one of the duties listed in this section. The lift director may assist in rigging or signaling. WAC 296-155-53401

Display the name(s) of the site supervisor, A/D director, lift director(s), rigger(s), signal person(s), and the crane/equipment operator(s) for the job site. The display shall be conspicuously posted on the premises for the entire duration that any crane/equipment is on-site.