Summary Discussion Leader: ___________________________ Attendance Sign- In: ____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

If you are driving a vehicle that is carrying a load or towing a trailer that is carrying a load, you have to make sure the load is properly secured. By neglecting to do this, you might cause an accident, injury or death as well as be legally liable.

Guide for Discussion

Help employees to understand that a few moments looking over a load and making sure it is properly secured can make a huge difference on the outcome of a successful or not so successful trip. Concerns are: - Objects fall from your vehicle onto other vehicles or pedestrians - Other drivers swerve to dodge items that are falling or have fallen from your vehicle onto the road - An unsecured load crashes into your vehicle cabin during emergency braking - A load shifts and contributes to your vehicle becoming unstable or unsafe - Articles 5 feet long or shorter weighing 1,100 pounds or less, require only one tie-down. - Articles 5 feet long or shorter weighing over 1,100 pounds require two tie-downs. - Articles longer than 5 feet need two and one every 10 feet - Separate items must have no space between them if not, individually secured. - Wheeled or tracked vehicles over 10,000 pounds require four direct (anchor) tie-downs and tightening devices. - Booms must be lowered and properly secured – preferably with an additional tie-down device. - Inspect straps for damage - Inspect trailer for proper hook up, ball size, pins and lights - Secure all loads, tarps or nets may be needed. - Anything that overhangs back by 4 feet needs flagging Always do a 3600 walk around before commencing with trip.

Additional Discussion Notes:

._______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Remember

If something falls off the trailer or truck, the driver is responsible, not the person who loaded the trailer.